DI22001-Algorithms and Artificial Intelligence

Delivery-Robot

Group 7

Maming Fu 2666035

Ruihang Gao 2666078

Yibo Xu 2666086

Feifan Liu 2666215

1.Introduction

With the rapid advancement of smart campus initiatives, autonomous delivery robots have emerged as a pivotal solution to streamline logistics operations, reduce human labor costs, and enhance the efficiency of daily goods distribution across university campuses. Central South University, like many large-scale academic institutions, features a sprawling campus with complex road networks, diverse pedestrian pathways, and a mix of residential, academic, and service-oriented areas. Navigating this intricate environment efficiently requires a robust path-planning system that can account for real-world spatial constraints, road characteristics, and optimal routing objectives.

This project focuses on developing a Minimum Viable Product (MVP) for path planning of campus delivery robots at Central South University, leveraging OpenStreetMap (OSM) data to model the campus road network and implementing classic AI search algorithms to generate optimal delivery routes. The solution addresses the core challenge of enabling autonomous robots to traverse the campus safely and efficiently, while providing transparent metrics to evaluate route performance and algorithmic effectiveness.

2.Problem Definition

This project focuses on the path planning problem for campus delivery robots at Central South University, which is a typical state-space search problem covered in the module. In real-world scenarios, large campuses have complex road networks with various path types, restricted areas, and different traversal difficulties. Manual delivery is inefficient and costly, while autonomous robots require safe and efficient routes between user-defined start and goal points. Real constraints include mixed road types, non-traversable sections such as steps and private areas, limited named points of interest, and possibly incomplete map data.

Formally defined under the module’s AI problem framework, this is a weighted graph optimal path search problem. States are geographic nodes; the initial state is the start location; the goal state is the destination; actions represent movement between adjacent nodes; and path cost combines distance and road-type multipliers. The goal is to compute valid and low-cost routes using three module search strategies: BFS, Uniform Cost Search, and A* Search. This problem supports the analysis of completeness, optimality, time complexity, and space complexity of uninformed and informed search algorithms.

3.Data

We use OpenStreetMap XML data exported from the Central South University campus area.

The OSM file contains:

Nodes: ~2,000–5,000 points with id, lat, lon, and OSM tags (names, amenities, buildings)

Ways: Ordered sequences of node references representing roads, footpaths, and service roads.

The OSM data is parsed into two primary data structures. First, a dictionary of nodes mapping each OSM node ID to its latitude, longitude, and associated tags (e.g., building names, amenities). Second, a list of ways, where each way contains an ordered sequence of node references representing a continuous road or footpath, along with its highway type and name.

These structures are then used to construct a weighted undirected graph using NetworkX. Each graph node stores an OSM ID and geographic coordinates. Each edge between two adjacent nodes stores the Haversine distance (in meters), the road type, and a computed cost value where cost = distance × road_type_multiplier. The multipliers range from 1.0 for footways to 1.5 for unclassified roads, reflecting preference for pedestrian-friendly routes. This graph serves as the input to all three search algorithms.

4.Approach

We implemented three search algorithms from scratch in ‘algorithm.py’.Each algorithm receives a ‘networkx.Graph’ with edge ‘cost’ attributes and returns path, total cost, expanded node count, and runtime.

(1)Breadth-First Search (BFS)

BFS ignores edge weights and explores in increasing hop count using a queue. It finds the path with the fewest edges, which is not necessarily the shortest distance or lowest cost. BFS is complete for finite graphs and optimal for unweighted graphs (minimising steps), but not for our weighted cost model.

(2)Uniform Cost Search (UCS)

UCS expands nodes in order of cumulative edge cost using a priority queue. It guarantees the optimal (lowest-cost) route for non-negative edge weights. UCS is complete and optimal, but may expand many nodes before reaching the goal, especially when many paths have similar costs.

(3) A* Search

A* uses the evaluation function f(n)=g(n)+h(n) where:

g(n)= cumulative edge cost from start to n

h(n)= straight-line Haversine distance from n to goal

h(n) is admissible (never overestimates true cost) because Euclidean distance ≤ any actual path distance on a plane. A* is therefore complete and optimal. The heuristic guides the search toward the goal, typically expanding fewer nodes than UCS.

(4) Additional Implementation Details

5. Results

Real-world interpretation: For a battery-powered delivery robot, A* provides the best balance: optimal cost with low computational overhead. UCS is suitable when precise optimality is required and computation time is not critical. BFS should be avoided for weighted routing but serves as a pedagogical baseline.

6. Future Enhancements

Despite a functional MVP, several improvements are possible:

(1) Dynamic cost updates:

Incorporate real-time factors – weather (rain increases path friction), crowd density (peak hours), or battery state (prefer flatter routes).

(2) Multi-goal routing:Extend search to support delivery to multiple buildings in optimal sequence (travelling salesman problem).

(3) Constraint Satisfaction:Use CSP to schedule delivery time windows (e.g., "deliver between 10:00–11:00") alongside path planning.

(4) Larger graphs: Our OSM data is campus-sized (~5,000 nodes). For city-scale routing, implement bidirectional A* or contraction hierarchies to maintain interactive performance.

(5) 3D terrain:Campus includes stairs and slopes. Integrate elevation data (from OSM or DEM) into edge cost.

7. Conclusion

This project successfully applied three AI search strategies (BFS, UCS, A) to campus delivery robot path planning using real OpenStreetMap data. We built a complete pipeline: OSM parsing, weighted graph construction, POI extraction, algorithm implementation, and an interactive Streamlit visualisation. A outperformed UCS and BFS in both cost optimality and computational efficiency, making it the most suitable for real-time robot navigation. The work demonstrates how classic graph search algorithms remain highly relevant for modern robotics applications. Future work can extend the cost model, add dynamic constraints, and scale to larger urban environments.

8. Reference